# Written Respiratory Protection Program

# Template & Guide

# Introduction

Respirators are used to protect employees from inhaling hazardous chemicals in the air. These chemicals can be in the form of gases, vapors, mists or dust.  If you provide respirators to your employees to protect them from airborne chemical hazards, you must have a written respiratory protection program.  The written program must spell out how you do the following at your workplace:

- How the proper respirators for the particular hazards are selected and issued (include a list of respirators used),

- When and how respirators will be used in routine work activities, infrequent activities, and foreseeable emergencies such as spill response, rescue or escape situations,

- How medical evaluations of respirator wearers is provided,

- How respirator fit-testing is done,

- How respirators in use are cleaned, stored, inspected and repaired or discarded,

- How sufficient high purity air is provided for air-supplied respirators (if you use them),

- How employees are trained about respiratory hazards at your workplace,

- How employees are trained on the proper use of the respirators used at your workplace,

- How you evaluate the effectiveness of your respiratory program.

The answers to the above “how” questions will depend on the unique conditions at your workplace.  The information must be specific and reflect what you actually do or require to be done, not just what seems like the right thing to do. It must describe actual conditions and actions at your workplace.

To provide proper protection, respirators must be the right type, must be worn correctly at all times, and must be maintained properly.   They are prone to leakage, depend on the correct behavior of individual employees and may require much maintenance and management oversight. This is why they are considered as a last resort to protect employees from airborne chemical hazards.

It often more protective, less trouble, and even cheaper to eliminate or reduce the respiratory hazard through various ways like exhaust ventilation, changes in process, or enclosure of the process.  Sometimes the use of a hazardous chemical itself can be eliminated.  But, when there is no alternative, a respirator program must be implemented to protect your employees from adverse health effects of exposure to chemicals in the air above their permissible exposure limits.

Respirators are typically used in three different situations – routine or regular exposure to processes or activities involving chemicals, infrequent, but predictable occasions where there is chemical exposure, or emergencies where there is a chemical leak or spill.  The written respiratory program must address all these situations if they occur or could occur at your facilities.

If you allow employees to wear respirators (either their own or those you provide), but don’t require their use, see the Rule requirements on voluntary use of respirators.  An editable sample voluntary respirator use program is also available.

The WISHA Respirators Rule requires that you designate one person as the “program administrator” who is responsible for the whole program. This person should be reasonably knowledgeable about chemical exposure, respirators and their uses and limitations and will need to keep track of respirator fit-testing, use, storage, cleaning and maintenance.  Don’t give this job to an untrained or unmotivated employee.  Training for persons acting as respirator program administrators is available from the University of Washington OSHA Training Institute or from respirator vendors.

The following sample respirator program will meet the WISHA requirements for a written program if all the blanks are filled in and it reflects actual practices at your worksite.

By clicking on underlined blue text, you can jump to the page containing the form or information indicated.

This sample respirator program can be edited on a computer. By filling in required information indicated in (red) in parenthesis or by the grey bar next to a line     _______________________ and printing the sample program, you will be stating the actual practices and procedures at your workplace.  Be sure to read the complete sample and make sure what is stated is what you actually do.  Some attached forms must be filled in by hand.

# Respiratory Protection Program for

(name of your company)

Our respirator program administrator is      _________________________________

Our administrator’s duties are to oversee the development of the respiratory program and, make sure it is carried out at the workplace.  The administrator will also evaluate the program regularly to make sure procedures are followed, respirator use is monitored and respirators continue to provide adequate protection when job conditions change.

#### Selection Of Respirators

We have evaluated our use of chemicals at this facility and found respirators must be used by employees in the following locations or positions or doing the following duties, tasks or activities:

| Employee position or activity | Chemicals or products used | ## NIOSH approved respirators assigned | ### When used (routinely, infrequently, or in emergencies) |
| --- | --- | --- | --- |

Find how to select proper respirators for particular chemical exposures

Find information on how to evaluate employee exposure to chemicals.

We selected these respirators based on the following information: [optional: attach air sampling results that show where respirators are required]

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

#### Medical Evaluations

Every employee of this company who must wear a respirator will be provided with a medical evaluation before they are allowed to use the respirator.  Our first step is to give the attached medical questionnaire to those employees.  Employees are required to fill out the questionnaire in private and send or give them to (name of your medical provider who will evaluate the questionnaire).  Our non-readers or non-English-reading employees will be assisted by (name of person not in management).  Completed questionnaires are confidential and will be sent directly to medical provider without review by management.

If the medical questionnaire indicates to our medical provider that a further medical exam is required, this will be provided at no cost to our employees by (name of medical provider doing medical exam) We will get a recommendation from this medical provider on whether or not the employee is medically able to wear a respirator.

Additional medical evaluations will be done in the following situations:

our medical provider recommends it,

our respirator program administrator decides it is needed,

an employee shows signs of breathing difficulty,

changes in work conditions that increase employee physical stress (such as high temperatures or greater physical exertion).

Find a copy of the required medical questionnaire in Chapter 296-842, WAC, Respirators.

Find information to provide to medical provider.

#### Respirator Fit-testing

All employees who wear tight-fitting respirators will be fit-tested before using their respirator or given a new one. Fit-testing will be repeated annually. Fit-testing will also be done when a different respirator facepiece is chosen, when there is a physical change in an employee’s face that would affect fit, or when our employees or medical provider notify us that the fit is unacceptable.  No beards are allowed on wearers of tight-fitting respirators.  Respirators are chosen for fit-testing following procedures in the WISHA Respirators Rule (Table 11) Fit-testing is not required for loose-fitting, positive pressure (supplied air helmet or hood style) respirators.   We do fit-testing using one or more of the following fit-testing protocols (circle protocol you use) or quantitative fit-testing instrument (fill in):

Irritant smoke protocol

Banana Oil (isoamyl acetate) protocol

Bitrex protocol

Saccharin protocol

The quantitative fit-testing instrument we use is:      _______________________________________________________________

Note: respirator vendors usually sell these instruments.  While expensive, they can simplify fit-testing.

Note: Click on the protocols above to view the several acceptable fit-testing procedures that come directly from the respiratory protection standard.  It is your choice which procedure or fit-testing equipment to use.  Some respirator vendors or occupational health clinics may do fit-testing for you.

Documentation of our fit-testing results is attached (or is kept at the following location)      _________________________________________________________________

Click here for a sample fit-test record form.

Our respirators will be checked for proper sealing by the user whenever the respirator is first put on, using the attached seal check procedures:

Click here for seal check procedures.

### Respirator storage, cleaning, maintenance and repair

Our non-disposable respirators will be stored in the following clean locations:

___________________________________________________________________________________________________________________________________________________________________________________________________________________

Respirators will be cleaned and sanitized every (     ____) days or whenever they are visibly dirty. (does not apply to paper dust masks which are disposed daily). Respirators will be cleaned according to the attached instructions (either the manufacturers instructions or the Respirators Rule cleaning procedures.)

For Rule-specified respirator cleaning procedures where you don’t have manufacturers instructions, click here.

All respirators will be inspected before and after every use and during cleaning.  In addition, emergency respirators and self-contained tank-type supplied air respirators in storage will be inspected monthly.

Respirators will be inspected for damage, deterioration or improper functioning and repaired or replaced as needed.  Repairs and adjustments are done by      __________ who is trained in respirator maintenance and repair. Supplied air respirators will be checked for proper functioning of regulator and warning devices and amount of air in tanks where used.

When supplied air respirators are used, any needed repairs or adjustments will be done by the manufacturer or technician trained by the manufacturer.  Our supplied air respirators are maintained and repaired by      ______________________________________

On respirators with vapor or gas cartridges, the cartridges will be regularly replaced on the following schedule:

Check with respirator vendor for recommended replacement schedule for each brand and type of respirator.

| ##### Type of respirator cartridge | Location or job duties | ##### Chemicals in use | ##### Replacement schedule |
| --- | --- | --- | --- |

###### Respirator Use

The Program Administrator will monitor the work area in order to be aware of changing conditions where employees are using respirators.

Employees will not be allowed to wear respirators with tight-fitting facepieces if they have facial hair (e.g., stubble, bangs) absence of normally worn dentures, facial deformities (e.g., scars, deep skin creases, prominent cheekbones), or other facial features that interfere with the facepiece seal or valve function.  Jewelry or headgear that projects under the facepiece seal is also not allowed.

If corrective glasses or other personal protective equipment is worn, it will not interfere with the seal of the facepiece to the face.

Note: Full-facepiece respirators can be provided with corrective glasses since corrective lenses can be mounted inside a full-facepiece respirator.  Contact lenses can also be used with full facepiece respirators if they do not cause any problems for the employee.

A seal check will be performed every time a tight-fitting respirator is put on.

The program administrator will make sure that the NIOSH labels and color-coding on respirator filters and cartridges remain readable and intact during use.

Employees will leave the area where respirators are required for any of the following reasons:

to replace filters or cartridges,

when they smell or taste a chemical inside the respirator,

when they notice a change in breathing resistance

to adjust their respirator,

to wash their faces or respirator,

if they become ill,

if they experience dizziness, nausea, weakness, breathing difficulty, coughing, sneezing vomiting, fever or chills.

The Program Administrator has identified the following areas or job duties as presenting the potential for IDLH (immediately dangerous to life or health) conditions:     __________________________________________________________________________________________________________________________________________________________________________________________________________

Where any area or confined space is designated as IDLH, we will provide one or two (circle) standby employees outside the area.  These standby employees are trained in effective emergency rescue, are equipped with pressure-demand self-contained breathing apparatus (SCBAs), and will be in constant visual, voice or signal line communication with the employees in the IDLH area.  The standby employees will notify the administrator before entering the IDLH area, and we will provide the necessary assistance when notified.

Click here for criteria for one or two standby employees.

### Breathing Air Quality for Supplied Air Respirators (if used)

Only Grade D breathing air will be supplied to compressed air tanks for respirators.

Click here for description of Grade D breathing air. (WAC 296-842-20005)

Our compressors used for breathing air supply are non-oil lubricated and the air intake is located in an uncontaminated area.  The brand name of our air compressor(s) and the location is as follows:

___________________________________________________________________________________________________________________________________________

Our compressors are equiped with filters, water traps and sorbents to provide clean, safe air.  They are maintained by      ________________________________  Maintenance records are located at      _________________________________________________

Optional:  We use oil-lubricated compressor(s) used for breathing air. These compressor(s) are equipped with carbon-monoxide alarms, high-temperature alarms or both. (circle one) They are located at     _____________________________________

Periodic carbon monoxide monitoring is done by      ______________________ on the following schedule on our compressor(s) with no carbon monoxide alarm:

________________________________________________________________________

If used, our airline respirators are equiped with air couplings that are not compatible with couplings to nonrespirable air (plant air for example) or other gas systems.

If used, our air cylinders for supplied air respirators are inspected and tested according to federal DOT regulations.

###### Respirator Training

Training is done by      ___________________________ before employees wear their respirators and annually thereafter as long as they wear respirators. Our supervisors or crew bosses who wear respirators or supervise employees who do, will also be trained on the same schedule.

Additional training will also be done when an employee uses a different type of respirator or workplace conditions affecting respiratory hazards or respirator use have changed.

Training will cover the following topics:

Why the respirator is necessary,

The respirator’s capabilities and limitations,

How improper fit, use or maintenance can make the respirator ineffective,

How to properly inspect, put on, seal check, use, and remove the respirator,

How to clean, repair and store the respirator or get it done by someone else,

How to use a respirator in an emergency situation or when it fails,

Medical symptoms that may limit or prevent respirator use,

Our obligations under the Respirators Rule.

Our training program is attached.

Click here for sample respirator training record form.

#### Respiratory Program Evaluation

We evaluate our respiratory program for effectiveness by doing the following steps:

- Checking results of fit-test results and health provider evaluations.

- Talking with employees who wear respirators about their respirators – how they fit, do they feel they are adequately protecting them, do they notice any difficulties in breathing while wearing them, do they notice any odors while wearing them, etc.

- Periodically checking employee job duties for changes in chemical exposure.

- Periodically checking maintenance and storage of respirators.

- Periodically checking how employees use their respirators.

- Other ____________________________________________________________

###### Recordkeeping

The following records will be kept:

A copy of this completed respirator program

Employees’ latest fit-testing results

Employee training records

Written recommendations from our medical provider

The records will be kept at the following location:     __________________________

_______________________________________________________________________

Employees will have access to these records.

End of Sample Program

# The following pages are informational pages or forms you can use as attachments to the sample program. Links to these are provided in the sample program.

# How to Select the Correct Respirator

The type and brands of respirators vary widely ranging from simple dust masks to supplied air respirators like the kind firemen wear.  Following is description of the main types of respirators.

#### Dust Masks (filtering facepieces)

These simple, two-strap disposable dust masks are designed only for dusts.  They are not as protective as other respirators, but do an adequate job in many cases, unless the dust is really toxic or copious. Don’t confuse these two-strap masks with the less protective one-strap dust mask designed only for pollen or non-toxic dust.

#### Half-Face Air-Purifying Respirator

These respirators are sometimes called “half-face” or “half-mask” respirators since they cover just the nose and mouth.  They have removable cartridges that filter out either dust, chemicals or both. Selecting the correct cartridges is essential since they are designed for particular types of chemicals or dust. A reputable respirator vendor can assist you in selecting the correct cartridges. These cartridges are typically removable and sometimes interchangeable.  Cartridges are available for solvents, ammonia, chlorine, acids and other chemicals.  The cartridges must be changed out or replaced periodically, especially for chemicals, since they can absorb only so much contaminant before breakthrough occurs.  A few cartridges are equipped with end-of-service indicators that show when a cartridge should be replaced.  Most cartridges don’t have this indicator and you must develop a change-out schedule to prevent breakthrough.  The change-out schedule is based on the chemical concentration, physical work effort, temperature and humidity.  Many respirator manufacturers have cartridge change schedule calculators available on the Internet.

#### Full-Face Air-Purifying Respirator

In some situations, you may need or want to use full-face respirators.  This type of respirator is used when the air contaminant irritates the eyes.  They also provide somewhat higher protection to the lungs since they tend to fit tighter and are less prone to leaking. These respirators also have replaceable cartridges that must be changed on a regular basis as described above for half-face respirators.

#### Powered Air Purifying Respirator (PAPR)

Powered Air Purifying Respirators have a battery pack that draws air through replaceable cartridges and blows into a full facepiece, helmet or hood. These respirators are often more comfortable in hot weather and some can provide more protection, depending on the type.  The cartridges must be changed regularily as describe for half-face respirators above.

Airline Respirator			Tank-type respirator (SCBA)

#### Supplied Air Respirators and Self-Contained Breathing Apparatus (SCBA)

In a few situations, you may need to provide a supplied air respirator to your employees. These situations include large chemical spills or leaks, entering a confined space where there is lack of oxygen or high levels of air contaminants, or working around extremely toxic chemicals. They may also be necessary working at hazardous waste sites, during sandblasting or in some spray painting operations. “Supplied air,” means that clean air is provided by means of an air hose from a compressor or a pressurized air tank.

Supplied air respirators are required when a respiratory hazard is considered “immediately dangerous to life or health” (also called “IDLH”).  Respiratory hazards are classified as IDLH as follows:

- There is a lack of oxygen (less than 19.5% oxygen)

- There is too much oxygen (more than 23.5% - a fire hazard)

- You know there are toxic chemicals in the air, but you don’t know how much

- The amount of chemical in the air is known or expected to be above the IDLH level for that chemical.  See the NIOSH Pocket Guide to Chemical Hazards for chemical IDLH levels.

Levels of chemicals above IDLH can occur in confined spaces, or enclosed spaces where there is little or no ventilation.

Emergency Escape Respirators

Emergency escape respirators, as the name implies, can only be used for one thing – to escape or exit from a room or building in an emergency, usually a large chemical release, leak or spill, or when a supplied air respirator fails or runs out of air.  An escape respirator is typically a small bottle or tank of air connected to a facepiece that supplies 5-10 minutes of air.  Some supplied air respirators will have an auxiliary bottle of air for escape that connects to the existing facepiece.

How do you decide which type of respirator to select?  First, it must be the correct type for the air contaminant.  Second, it must fit properly. Third, it must provide adequate protection for the amount of chemical in the air.  The more toxic or more concentrated the chemical is in the air, the higher the level of protection the respirator must provide.

Different respirators provide different protection.  Depending on the amount of chemical in the air, you may need to use a respirator that provides more protection. Respirators are rated by their “assigned protection factor” (APF) which is a number between 10 and 10,000.  The higher the number, the greater the protection.  A respirator with a protection factor of 10 will provide adequate protection to levels of the chemical in the air 10 times the safe limit of that chemical. See Table 5 below.

Use Table 6 below to select air-purifying respirators for particle, vapor, or gas contaminants.. Table 6 Requirements for Selecting Air-purifying Respirators. If the contaminant is a:; Then. Gas or vapor; Provide a respirator with canisters or cartridges equipped with a NIOSH-certified, end-of-service-life indicator (ESLI) (note: there just a few of these) or If a canister or cartridge with an ESLI is not available, develop a cartridge change schedule to make sure the canisters or cartridges are replaced before they are no longer effective (note: most cartridge respirators fit in this category) or Select an air-supplying respirator. Particle, such as a dust, spray, mist, fog, fume, or aerosol; Select respirators with filters certified to be at least 95% efficient by NIOSH. For example, N95s, R99s, P100s, or High Efficiency Particulate Air filters (HEPA) Or You may select respirators NIOSH certified as “dust and mist,” “dust, fume, or mist,” or “pesticides.” You can only use these respirators if particles primarily have a mass median aerodynamic diameter of at least 2 micrometers Note: These latter respirators are no longer sold for occupational use, but some employers may still be using them...

How to Evaluate your Workplace for Employee Exposure to Chemicals

Respirators are required when employees are exposed (can inhale) chemicals or dust in the air that are at harmful levels.  These can include vapors from handling solvents, spray-painting, dust from grinding or sanding, or welding fumes.  If you manage a small business, you probably quite familiar with each employee’s job, what chemicals they use or how much welding, spray painting, grinding or sanding they do.  Your employees may have told you that the chemical odors or dust bothered them or that they were worried about their chemical exposure.  You may have switched to less hazardous chemicals. Or you may have no alternative but to use more hazardous chemicals to do the job or make your product.  But without some knowledge of the amount of chemical or dust in the air in the workplace, you cannot know whether your employees are exposed to harmful amounts of chemicals they use.

Just about every chemical has its toxic amount or level that will make person sick.  Even too much table salt can be harmful.  On the other hand, highly toxic chemicals can be used without harm to employees if handled properly.  Most commonly used chemicals have safe limits or “permissible exposure limits” in the air that if exceeded will cause harm. To view the list of chemicals with WISHA permissible exposure limits, click here. If these limits are exceeded, you are required to take steps to protect your employees from that air exposure.  If the levels cannot be reduced below the permissible exposure limits by ventilation, changes in the process or reduction in the length of time of exposure, than you must provide respirators to exposed employees.

The best way to accurately determine the levels of chemicals or dust in the air is to do some type of air sampling.  There are a variety of instruments and devices for measuring air contaminants.  Some are simple and cheap, most are quite expensive.  The methods for doing the air sampling accurately are usually fairly complicated and should not be done by a layperson.  Air sampling can be done by WISHA industrial hygiene consultants at your request.  This is a free service and will not result in a citation or penalty or a report to WISHA safety inspectors.   To request this service, contact the nearest Department of Labor & Industries Office near you. Click here for contact information. You can also have a private industrial hygiene consultant conduct air sampling. They can be found under “industrial hygiene services” in the Yellow Pages.

If you belong to a trade association or industry group, that organization may have information on common chemical hazards and methods of controls.  The material safety data sheets for products used also provide information about the hazards of the chemicals, permissible exposure limits, methods of controls and recommended respirators.

Sample Respirator Fit Test Record

Name: __________________________________________________ Initials: ________

Type of qualitative/quantitative fit test used: _________________________________

Name of test operator: _____________________________________ Initials: _______

Date: _________________

Respirator Mfr./Model/Aproval no.		Size	Pass/Fail  or	Fit Factor

Note: “Fit factor” is numerical result of quantitative fit test from instrument reading

1. _______________________________________S M L	    P    F	  _____

2. _______________________________________S M L	    P     F	  _____

3. _______________________________________S M L	    P     F	  _____

4. _______________________________________S M L	    P     F	  _____

Clean Shaven?	  Yes___	No___ (Fit-test cannot be done unless clean-shaven)

Medical Evaluation Completed?	Yes___	No___

NOTES: _____________________________________________________________

____________________________________________________________________

____________________________________________________________________

____________________________________________________________________

This record indicates that you have passed or failed a qualitative or quantitative fit test as shown above for the particular respirator(s) shown.  Other types will not be used until fit tested.

# Sample Respirator Training Record

___________________________________________

Employee Name (printed)

I certify that I have been trained in the use of the following respirator(s):

This training included the inspection procedures, fitting, maintenance and limitations of the above respirator(s).  I understand how the respirator operates and provides protection.  I further certify that I have heard the explanation of the respirator(s) as described above and I understand the instructions relevant to use, cleaning, disinfecting and the limitations of the respirator(s).

__________________________________

Employee Signature

__________________________________

Instructor Signature

__________________________________

Date

#### Employer-Provided Information for Medical Evaluations

This form may be used by the employer to give to your medical provider, information on respirator use by your employees, but it is not a required form. You can also consult directly with your medical provider and discuss the information below.

You must also give the medical provider a copy of your written respiratory program and copy of the Respirators Rule

### Specific Respirator Use Information

Employee Name: ___________________________________________

Company name: _________________________________________________

Employee job title: __________________________________________

Company Address: _______________________________________________

Company contact person and phone #:_______________________________________________________

1.  Will the employee be wearing protective clothing and/or equipment (other than the respirator) when using the respirator?

Yes/No _______ If “Yes,” describe protective clothing and/or equipment:

______________________________________________________________________________________________________________________________

______________________________________________________________________________________________________________________________

2.  Will employee be working under hot conditions (temperature exceeding 77°F)?

Yes/No ________ If “Yes”, describe nature of work and duration:

______________________________________________________________________________________________________________________________

______________________________________________________________________________________________________________________________

3.  Will employee be working under humid conditions?  Yes / No_______

4. Describe any special or hazardous conditions the employee could encounter when using the respirator (for example, confined spaces, life-threatening gases).

________________________________________________________________________________________________________________________________________________________________________________________________________________________

### Specific Respirator Use Information, Continued

| Check Box | Respirator Type | Face / Head Cover Type (half or full face, helmet, or hood) | Frequency of Use (hours per day, week, or month) | Work Effort Light, Moderate, Heavy (see descriptions below) | Respirator Wt. |
| --- | --- | --- | --- | --- | --- |
|  | Disposable facepiece particulate filter (N, R or P series) | 1/2 facepiece |  |  |  |
|  | Mask with replaceable filter or cartridge |  |  |  |  |
|  | Mask with canister |  |  |  |  |
|  | Powered air-purifying respirator (PAPR) |  |  |  |  |
|  | Air line, continuous flow |  |  |  |  |
|  | Air line, negative pressure demand |  |  |  |  |
|  | Air line, positive pressure demand |  |  |  |  |
|  | SCBA, negative pressure demand | Full facepiece |  |  |  |
|  | SCBA, positive pressure demand | Full facepiece |  |  |  |

### Work Effort Descriptions

Examples of a light work effort are sitting while writing, typing, drafting, or performing light assembly work; or standing while operating a drill press (1-3 lbs.) or controlling machines.

Examples of moderate work effort are sitting while nailing or filing; driving a truck or bus in urban traffic; standing while drilling, nailing, performing assembly work, or transferring a moderate load (about 35 lbs.) at trunk level; walking on a level surface about 2 mph or down a 5-degree grade about 3 mph; or pushing a wheelbarrow with a heavy load (about 100 lbs.) on a level surface.

Examples of heavy work effort are lifting a heavy load (about 50 lbs.) from the floor to your waist or shoulder; working on a loading dock; shoveling; standing; standing while bricklaying or chipping castings; walking up an 8-degree grade about 2 mph; climbing stairs with a heavy load (about 50 lb.).

# Criteria for deciding whether one or two standby employees are needed at an IDLH location

WAC 296-842-19005

Provide standby assistance in immediately dangerous to life or health (IDLH) conditions

You must

• Provide at least 2 standby employees outside the IDLH area.

|  | Note: You need only one standby employee if the IDLH condition is well characterized, will remain stable and you can show one employee can adequately do all of the following: – Monitor employees in the IDLH area – Implement communication – Initiate rescue duties. |
| --- | --- |

Seal Check Procedures (from Respirators Rule)

Respirator Cleaning Procedures (from Respirators Rule)